### 1. Why modified z instead of a standard z-score?

With ~20 subjects, a single extreme value inflates the mean and standard deviation enough to hide itself. Median and MAD are resistant to that, so the modified z is the right screen at this sample size. The 0.6745 constant scales MAD to be consistent with the standard deviation under normality.

### 2. Why the threshold of 3.5?

Iglewicz and Hoaglin's original recommendation for the modified z. It's a conventional cutoff, not something I tuned to my data. If I'd chosen the threshold after seeing results, that would be a problem — but it's pre-specified from the method's source.

### 3. Why within-subject percent change from the still scan?

It controls for between-subject anatomical variability. Comparing each subject to their own still scan isolates the motion effect from baseline differences in brain size and thickness. This follows the within-subject logic of Reuter et al. (2015).

### 4. Why these six measures?

They span cortical thickness and subcortical volume.

### 5. Why record Tukey and ENIGMA but not use them as exclusion rules?

Stacking multiple exclusion criteria inflates false positives and lets you manufacture a cleaner result. Recording them as corroboration gives the reader context without letting them drive decisions. One pre-specified primary screen is the inclusion rule.

### 6. Why do flags not change inclusion?

A quantitative flag isn't a data quality failure. Scans can have an extreme values for real biological reasons, because the still scan was unusual, or because of a segmentation error. The only way to know is to look. So the flag sends it to re-review; the written checklist determines inclusion. This is directly motivated by Reuter et al.'s finding that motion bias survives QC exclusion — the flag isn't the same as the fail.

### 7. Why separate long-format and wide-format outputs?

Long format preserves every subject × condition × measure contrast for detailed inspection and re-analysis. Wide format gives the subject × condition summary that's readable at a glance. Both serve different purposes.

### 8. Why the heatmap?

Your answer: It's a visual QC summary. One panel per motion condition, six measures on the x-axis, subjects on the y-axis, modified z as color, flagged cells marked with an asterisk. It lets a reviewer see the pattern across the sample in one view rather than reading a table.