## Replication Context

This project is a partial replication of:

Reuter M, Tisdall MD, Qureshi A, Buckner RL, van der Kouwe AJW, Fischl B.

Head motion during MRI acquisition reduces gray matter volume and thickness

estimates. NeuroImage. 2015;107:107-115.

doi:10.1016/j.neuroimage.2014.12.006

### Original study

Reuter et al. scanned 12 healthy volunteers within a single session under

different motion conditions (still, nod, shake, free motion). Using FreeSurfer

5.3, VBM8 SPM, and FSL Siena 5.0.7, they found an average apparent volume

loss of roughly 0.7%/mm/min of subject motion, with effects varying across

regions. Critically, the bias remained significant after excluding scans

that failed a rigorous quality check — motion bias is directional, not just

added noise.

### This project

- Dataset: MR-ART (OpenNeuro ds004173), 20 subjects × 3 motion conditions

(still, mild, moderate)

- Software: FreeSurfer 8.2.0 (vs. 5.3 in the original)

- Estimand: within-subject percent change from the still scan

- QC: pre-specified outlier screen (Iglewicz–Hoaglin modified z on percent

change) plus a written surface/WM/topology checklist

- Separation of flags (re-review) from fails (checklist-based exclusion),

motivated by the original finding that bias survives QC exclusion

### What's different

- Larger sample (20 vs. 12)

- Condition labels rather than mm/min motion quantification — effect sizes

are not directly comparable

- FreeSurfer-only (original tested three pipelines)

- Pre-specified measures (six locked ROIs) rather than whole-brain

morphometry

- Explicit flag-vs-exclusion documentation

### What replicates

- Direction of effect: motion → thinner cortex, smaller volumes

- Regional variation: temporal pole shows the largest drift in this sample

(−20.9% median), consistent with regional specificity in the original

- QC exclusion does not eliminate the bias